# Running LaMP

# Purpose

This document provides the basic steps for running a normal LaMP production probe on PROBE08 (or the applicable bench), using an already-built recipe. It does not cover creating or editing a recipe - see the Full GUI Manual for that.

# Scope

This procedure is for any personnel running LaMP wafers who do not need to build or modify the recipe itself.

# Equipment/Tooling

• PROBE08 Accretech UF200R Fully Automatic Prober (or applicable bench)

• LaMP program-specific probe card

# Procedure

## Load the ATA Folder and Recipe

1. Open the Atomica Tester executable. Verify instruments are connected (green checkmarks) and the working directory is correct.

2. Load the LAMPATA ATA folder (toolbar ATA Folder dropdown, or Load ATA Folder if it is not already listed).

3. Go to the Recipe tab and load the correct recipe for the probe card installed. For example, LaMP Electrical for standard production, or LaMP Electrical Gauge for a gauge/qualification wafer.

NOTE: If this bench always runs the same recipe, it can be set as the default recipe for this ATA folder/probe card so it loads automatically the next time this folder is opened, instead of needing to be reselected every run.

4. Verify the probe card selected on the Recipe tab matches the label on the physical probe card installed on the prober.

## Run

5. Go to the Run tab and load the recipe (Load Recipe button).

6. Press Full Die to test every die on the wafer map, or Test Die to test a selected subset of dies.

7. Probing will proceed automatically. The wafer map squares highlight green (pass) or red (fail) as each die completes.

8. Once probing is complete, press Unload under the Run tab, then remove the wafer per the normal Accretech (or Electroglas) unload procedure.

## Save Results

9. Go to the Results tab. Enter the Lot ID and/or Wafer ID before saving.

10. Verify the export path is the one expected for this project, then save (Save to CSV, or the project's own configured export format if one exists for LaMP).

# Records

\\prober\ETL\RAWDATA\PROBE08 (or the applicable export path for this bench/project)

# Attachments

(none)